# VxD Test Results

Driver:   PV5018C

| Feature | Description |
| --- | --- |
| First Installation | Initial Driver Installation |
| Mixer Controls | Input Mixer Control |
|  | Output Mixer Control |
| Wave Playback | Supports 5, 6, 8, 9, 11, 16, 18, 22, 27, 32, 33, 37, 44, 48 kHz sample rates |
| Wave Capture | Supports 5, 6, 8, 9, 11, 16, 18, 22, 27, 32, 33, 37, 44, 48 kHz sample rates; |
| Full Duplex Audio |  |
| DirectSound/DirectSound3D |  |
| Windows FM Synthesis |  |
| DirectShow |  |
| Joystick | The following Joysticks are working in Windows and in Dos Box: - MS Sidewinder Force Feedback Pro: - MS Sidewinder GamePad: (Single and Daisy Chain) - MS SideWinder Pro: d. 2-axis 2 button Analog Joystick: (Kraft Joystick) - Logitech Digital Extreme - Logitech Wingman The test consisted on boot up detection, hot plug detection, and detection after suspend/resume. The joysticks all worked well except for not being detected after a suspend/resume. |
| Advanced Power Management |  |
| General Dos Box Sound |  |
| General Dos Box Music |  |
| Dynamic Settings Changes |  |
| External Midi Input |  |
| External Midi Output |  |
| EEPROM Values | CFL1, SSVID, SSHwID. |
| Native Dos Game Testing |  |
| S/Pdif | AC-3 Data Output |
| Crystal 3D |  |
| AutoTranslations |  |
| Clock Run Support |  |
| Hardware Volume Control |  |

Limitations:

- Rev C includes FM Synthesis.   Therefore, CrystalWave will not be provided.

- Slight zipper noise with sndvol32 main and mp2 vol; not with wave nor synth Bug #3743

- Half-Life menu buttons noisy after brief gameplay (not before)

- Remove and Refresh Crystal CS4281 PCI Audio from Device Manager defaults to CS4281 External Midi. Bug #3733

- We have seen the system intermittently hang on shutdown when Novel Client 3.2 is present.

Bug Fixes

1) Wave audio pops.. please see details Bug #3727

Causes a loud pop for a drag and drop of a wave file on to a Media Player 2 also causes a pop for a Stop while playing a wave file.

2) Audio sounds different between an AVI and a Wave file Bug #3729

Play an AVI thru MP2 and Play a wave thru another MP2, AVI sounds mono, close wave and focus on AVI, it sounds normal, try vice versa, it happens the same.

3) Opening any Dsound application causes loud pops Bug #3731

4) Severe static on occasion when changing DVD tracks (through S/PDIF using PowerDVD) Bug #3750

5) MS Sidewinder 3D Pro joystick not detected on K6-200MHz Bug #3756

6) mplayer2 pops with wav, mp3, mpg when stop, sometimes when play via drag & drop Bug #3847

- Clock Run Addition

- Bug #3869: No Redbook audio in DOS Box; must touch CD mixer to hear in Windows after DOS Box app.

- Bug #3870:  We are not able to remove sliders.  AC97SourcesUsed is not working.

- Modify MonoIn table values to fix a problem with PC BEEP

- Move Joystick speed setting, hardware master volume enable,

codec configuration and SPDIF enable flag to the inf file

12) Add registry flag indicating that Legacy should use

the PCI interrupt instead of enumerating one as we

currently do, for customers wanting to save some

resources.

13)  change inf to allow the possibility of loading the loader at the root   HKLM,%KEY_MSDOS%\CS4281,Autoexec.Bat,,"%%WINDIR%%\cwcdata\cwrdos.exe"

14)  chane cwrgal.dat file to remove  HKLM,%KEY_MSDOS%\CS4281,Autoexec.Bat,,"%%WINDIR%%\cwcdata\cwrdos.exe"

15) Change joystick OEM callback data

16) Turn S/pdif off by default.